Table of Contents

- Overview

- Installation

- Getting Started

- Plugin Controls

- Technical Specifications

- Usage Scenarios

- Tips and Best Practices

- Troubleshooting

- Support

# Overview

HAAS-49 is a professional stereo enhancement plugin designed to transform mono audio sources into rich, wide stereo soundscapes. The plugin combines advanced frequency processing with psychoacoustic stereo widening techniques to create natural-sounding stereo imagery from mono input signals.

Key Features

- Mono to Stereo Conversion: Intelligently converts mono signals to stereo

- Independent Channel Processing: Separate 3-band EQ for left and right channels

- Haas Effect Processing: Psychoacoustic delay-based stereo widening

- Stereo Width Control: Adjustable stereo field expansion

- Output Level Management: Precise gain control with visual feedback

Applications

- Stereo enhancement of mono recordings

- Creative stereo field manipulation

- Mix bus processing for width enhancement

- Mastering applications for stereo imaging

- Live performance stereo enhancement

InstallationSystem Requirements

- Windows: Windows 10 or later (64-bit)

- DAW: Any VST3-compatible host application

- Memory: Minimum 4GB RAM recommended

- Storage: 10MB available disk space

Installation Steps

- Download: Obtain the HAAS-49.vst3 or desired version of the plugin from Unsound Studios in the .zip

- Locate VST3 Folder: Navigate to your VST3 plugin directory:

  - User Directory: `%USERPROFILE%\\AppData\\Roaming\\VST3\\`

  - System Directory: `C:\\Program Files\\Common Files\\VST3\\` (requires admin rights)

  - MacOS: ~/Library/Audio/Plug-Ins/Components

  - MacOS: ~/Library/Audio/Plug-Ins/VST3/

- Install Plugin: Copy `HAAS-49.vst3` or ‘.compontent’ to the VST3/Component directory

- Refresh DAW: Restart your DAW or refresh the plugin list

- Verify Installation: Look for "HAAS-49" in your plugin browser under "Unsound Studios"

DAW-Specific Notes

- Reaper: Use "Rescan" in the FX browser

- Pro Tools: Refresh AAX plugins (if AAX version available)

- Logic Pro: Use "Reset & Rescan Selection" in Plugin Manager

- Cubase/Nuendo: Use "Update Plugin Information" in Plugin Manager

- Studio One: Use "Reset Blacklist" in Plugin Manager

Getting Started

Basic Workflow

- Load Plugin: Insert HAAS-49 on a mono track or bus

- Set Input: Ensure mono signal is feeding the plugin

- Adjust Haas Effect: Start with Delay around 15-25ms and Amount at 50%

- Balance Channels: Use the EQ controls to differentiate left/right frequency content

- Fine-tune Width: Adjust Stereo Width to taste (typically 100-150%)

- Set Output Level: Use Output Gain to match bypass level

Quick Start Presets

While HAAS-49 doesn't include presets, these starting points work well:

Subtle Enhancement:

- Haas Delay: 12ms, Amount: 30%

- EQ: Minimal adjustments (±2dB)

- Stereo Width: 120%

Dramatic Widening:

- Haas Delay: 25ms, Amount: 70%

- EQ: Contrast left/right channels (±5-8dB)

- Stereo Width: 160%

# Plugin Controls

Haas Effect Section

Haas Delay (0-50ms)

Controls the delay time applied to create the Haas effect. The Haas effect is a psychoacoustic phenomenon where slight delays between left and right channels create perceived stereo width without distinct echoes.

Usage Guidelines:

- 5-15ms: Subtle width enhancement, maintains mono compatibility

- 15-25ms: Moderate stereo expansion, good for most sources

- 25-40ms: Wide stereo field, may affect mono compatibility

- 40-50ms: Extreme width, use sparingly

Haas Amount (0-100%)

Determines the intensity of the Haas effect processing.

Usage Guidelines:

- 0-30%: Gentle enhancement, preserves original character

- 30-60%: Moderate processing, noticeable stereo expansion

- 60-80%: Strong effect, significant width increase

- 80-100%: Maximum processing, dramatic stereo field

Channel EQ Section

Each channel (Left/Right) features an independent 3-band equalizer:Low Band (250Hz, ±20dB)

High-pass filter centered at 250Hz for low-frequency control.

Applications:

- Remove unwanted low-end rumble

- Create channel-specific bass response

- Improve mix clarity in low frequencies

Mid Band (1kHz, ±20dB)

Peaking filter at 1kHz affecting vocal and instrumental presence.

Applications:

- Enhance vocal clarity and intelligibility

- Adjust instrumental presence

- Create frequency contrast between channels

High Band (4kHz, ±20dB)

High-frequency shelving filter at 4kHz for brilliance and air.

Applications:

- Add sparkle and air to recordings

- Enhance cymbal and percussion clarity

- Create channel brightness difference

Output Section

Stereo Width (0-200%)

Controls the overall stereo field width of the processed signal.

Usage Guidelines:

- 0-50%: Narrow stereo field, approaching mono

- 50-100%: Normal to slightly enhanced width

- 100-150%: Enhanced stereo field, good for most applications

- 150-200%: Very wide stereo field, use with caution

Output Gain (±20dB)

Final output level control with visual level indication.

Applications:

- Compensate for level changes during processing

- Match bypass levels for A/B comparison

- Prevent output clipping

# Technical Specifications

Audio Specifications

- Sample Rate Support: 44.1kHz to 192kHz

- Bit Depth: 32-bit floating point processing

- Latency: Less than 5ms at 44.1kHz

- Dynamic Range: >120dB

- THD+N: <0.01% at nominal levels

Processing Details

- EQ Type: High-quality IIR filters

- Delay Processing: Sample-accurate delay lines

- Oversampling: Internal 2x oversampling for critical processing

- CPU Usage: Optimized for real-time performance

Plugin Format

- Format: VST3

- Architecture: 64-bit native

- Parameters: 8 automatable parameters

- Preset Support: Host-dependent preset management

# Usage Scenarios

Mono Instrument Enhancement

Application: Electric guitar, bass, keyboards, vocals

Approach: Use moderate Haas settings (15-20ms, 40-60%) with subtle EQ differences between channels.Drum Processing

Application: Mono drum samples or room mics

Approach: Shorter Haas delays (8-15ms) with frequency-specific channel processing to create realistic stereo drum images.Mix Bus Processing

Application: Stereo bus enhancement

Approach: Very subtle settings (5-10ms delay, 20-40% amount) to add width without disrupting existing stereo information.Creative Sound Design

Application: Atmospheric textures, sound effects

Approach: Extreme settings with high contrast EQ between channels for unique stereo textures.Live Performance

Application: Real-time mono source enhancement

Approach: Conservative settings to ensure mono compatibility for FOH systems while providing stereo enhancement for recording feeds.

# Tips and Best Practices

Mono Compatibility

- Test your processed audio in mono to ensure no phase cancellation

- Shorter Haas delays (under 15ms) generally maintain better mono compatibility

- Use the Stereo Width control to reduce width if mono issues arise

Level Management

- Monitor output levels closely, especially with high Stereo Width settings

- Use the Output Gain to compensate for perceived level changes

- Consider the cumulative effect of multiple instances in a mix

EQ Strategy

- Create subtle frequency differences between channels rather than dramatic boosts/cuts

- Consider the frequency content of your source material when adjusting EQ

- Use high-frequency differences to enhance stereo perception

Automation

- Automate Haas Amount for dynamic width changes during different song sections

- Use Stereo Width automation for creative mix movements

- Consider automating Output Gain for consistent levels across different processing intensities

Signal Chain Placement

- Before Compression: For natural dynamic response to width changes

- After EQ: To maintain intended frequency balance

- On Sends: For parallel processing and blend control

# Troubleshooting

Common Issues

No Stereo Effect Perceived

Possible Causes:

- Monitoring in mono

- Input signal already stereo

- Haas Amount set too low

Solutions:

- Verify stereo monitoring setup

- Ensure mono input signal

- Increase Haas Amount gradually

Phase Issues in Mono

Possible Causes:

- Excessive Haas delay times

- High Stereo Width settings

- Extreme EQ differences between channels

Solutions:

- Reduce Haas Delay to under 15ms

- Lower Stereo Width percentage

- Minimize EQ differences between channels

CPU Performance Issues

Possible Causes:

- Multiple instances running simultaneously

- Very high sample rates

- Insufficient system resources

Solutions:

- Bounce processed tracks to audio

- Reduce sample rate if possible

- Close unnecessary applications

Plugin Not Loading

Possible Causes:

- Incorrect installation directory

- DAW not recognizing VST3 format

- File corruption during download

Solutions:

- Verify VST3 directory path

- Check DAW VST3 support

- Re-download and install plugin

Performance Optimization

- Use plugin instances strategically rather than on every track

- Consider bouncing heavily processed tracks to audio

- Disable plugin instances on muted tracks when possible

# Support

Technical Support | For technical issues, compatibility questions, or bug reports:

- Email: support@unsound-studios.com

- Response Time: 24-48 hours for technical inquiries

Feature Requests | To suggest new features or improvements:

- Email: feedback@unsound-studios.com

- Discord: Visit our user community at discord.unsoundstudios.com

Documentation Updates | This manual is updated regularly. Check for the latest version at:

- Website: www.unsound-studios.com/product-pages/haas-49

- Version: Current manual version 1.0

Additional Resources

- Video Tutorials: Available on our YouTube channel

- User Discord Community: Join discussions with other HAAS-49 users

- Update Notifications: Subscribe to our newsletter for plugin updates